|  | Workshop 3-Part I Basic principles of uncertainty analysis | T-316-LABB Fall 2023 |
| --- | --- | --- |

Preliminaries

# Today’s workshop is about:

Learning the terminology of uncertainty analysis

Distinguishing between systematic and random uncertainty

Estimating systematic, random, and total uncertainty in a resultant

Applying two rules of thumb for reporting a result

Putting these skills and knowledge into practice

Preparing you for the complete problem you will perform next period

# Please learn to define the following terms.

| measurand |  |
| --- | --- |
| sensor, transducer (often used interchangeably) |  |
| confidence interval |  |
| data reduction equation |  |
| systematic uncertainty |  |
| random uncertainty |  |
| total uncertainty |  |
| RSS |  |
| error propagation |  |
| sensitivity coefficient |  |

# The data reduction equation relates the measurands to the resultant.

The goal of an experimental program is to determine a resultant R and its uncertainty wR,

|  | . |  |
| --- | --- | --- |

The data reduction equation (DRE) is the function (or set of functions) that relates the n measurands (x1, x2, …, xn) to the resultant R, that is,

|  | . | (1) |
| --- | --- | --- |

Consult with your instructor before deciding to combine a set of simple DREs into a single but unnecessarily complex DRE.

# All uncertainties are either systematic or random.

Systematic uncertainty : Systematic uncertainty is a repeatable uncertainty in a measurement that does not vary with replication.  It is also known as “bias”.  It is estimated by the difference between the true value of a measurand and the average of a large number of readings.  An uncertainty is systematic if and only if it can be improved by calibration. Typical elementary sources of systematic uncertainty are the accuracy and readability of a sensor.

Random uncertainty : Random uncertainty is a non-repeatable error  due to unknown or uncontrollable factors influencing the measurement.  An uncertainty is random if and only if it can be improved by running additional trials. Typical sources of random uncertainty are small fluctuations in a steady-state measurand in the instrumentation, or in environmental conditions.

Figure 1: Systematic uncertainty B (“bias”) and range of random uncertainty P (“precision”).  The true value is never known.

# Total uncertainty.

Total uncertainty is the root of the sum of the squares (RSS) of the systematic and random uncertainties,

|  | . | (2) |
| --- | --- | --- |

It is convenient to square both sides of (2) and use the following form:

|  | . | (3) |
| --- | --- | --- |

Our goal today is to define all the terms needed to derive & evaluate (3).

Detailed uncertainty analysis

# Flow chart

The flow chart shows the systematic and random uncertainties commonly encountered in experimental work. The arrows indicate how the sources of uncertainty are computed and combined to produce the total uncertainty estimate for the resultant.

Reminder:  that the goal of an experiment is to determine the resultant with uncertainty,

.

The flow chart is a roadmap to estimating .

|  | Total uncertainty in the resultant |
| --- | --- |

| Systematic uncertainty in the resultant |  |
| --- | --- |

| Systematic uncertainty in the measurand |  |
| --- | --- |

| Random uncertainty in the resultant (3 options) |  |
| --- | --- |
| 1. multiple measurements |  |
| 2. single measurement |  |
| 3. no SR feasible |  |

| Random uncertainty in the measurand, used for option 3 only |  |
| --- | --- |

Figure 2. Commonly-encountered components of a detailed uncertainty analysis.

In subsequent sections, each element of the flow chart is developed in detail. By the end of this lesson, you should be able to compute and describe each element of this flow chart.

# Systematic uncertainty in a measurand

The two most commonly encountered systematic uncertainties are due to accuracy and readability.

- Uncertainty due to accuracy is ± the value obtained from a manufacturer’s specification sheet or from a calibration (often a percentage). If unavailable, as a last resort only, accuracy can be assigned the same value as the uncertainty due to readability.

- Uncertainty due to readability is ± half the resolution in a digital readout or ± half the smallest division in an analog readout. If you see large fluctuations in the output for a steady-state measurand, then the uncertainty is ± half the least digit that is steady.

Total systematic uncertainty in a measurand is the RSS of all systematic uncertainties for that measurand. Two cases are commonly encountered:

| - A sensor with an separate readout (such as a DMM), |  | (4) |
| --- | --- | --- |
| - The sensor & readout are combined into a single unit |  | (5) |

# Systematic uncertainty in the resultant

The measurand uncertainties affect the resultant uncertainties via the error propagation relationship,

|  |  | (6) |
| --- | --- | --- |

where xi is a particular measurand, n is the number of measurands, and the function R(x) comes from (1).

The partial derivative terms in (6) are called sensitivity coefficients because their magnitudes indicate how sensitive the resultant uncertainty is to the measurand uncertainties.

- If this coefficient is large, the sensor accuracy or readability might have to be improved, possibly by using different instrumentation.

- If this coefficient is very small, the sensor may not require high accuracy or readability.

- Because of the partial derivatives, the sensitivity coefficients are functions of the measurands. Because measurands are not always constant, the sensitivity coefficients may have to be computed separately at each operating condition or test point.

# Random uncertainty

Random uncertainty is usually estimated in one of three ways:

- Multiple trials at an operating condition, allowing the estimation of a mean resultant and a standard deviation SR in the resultant.

- A single trial at an operating condition produces a resultant. A separate experiment is run to estimate the standard deviation SR of the resultant.

- The nature of the experiment does not permit an estimate of the standard deviation of the resultant, so the random uncertainty must be estimated in the measurands instead.

## Multiple trials (random uncertainty option 1)

With multiple measurements of a steady-state measurand, random uncertainty in the resultant is

|  | , | (7) |
| --- | --- | --- |

where SR is the standard deviation of the multiple measurements and n is the sample size. For the “t critical value” we typically use 95% confidence and a 2-sided interval. See Table 6.6.

## Single trial (random uncertainty option 2)

With a single measurement, the random uncertainty in the resultant is

|  | . | (8) |
| --- | --- | --- |

Of course you can’t estimate SR from a single measurement. You or someone else has to conduct an auxiliary experiment with multiple measurements to estimate SR.

## Random uncertainty in the measurands (random uncertainty option 3)

Suppose that the nature of the experiment or resultant does not permit an estimate of SR as required in (7) and (8). In this case, we derive a new model of random uncertainty, as follows. Total uncertainty is

|  |  | (9) |
| --- | --- | --- |

where the total uncertainty in each measurand is the RSS of both systematic and random uncertainties in that measurand, i.e.,

|  |  | (10) |
| --- | --- | --- |

Substituting (10) into (9) for every measurand xi and collecting the systematic and random terms yields

|  | . | (11) |
| --- | --- | --- |

The collected systematic term is identical to (6). The collected random term is our new model,

|  | . | (12) |
| --- | --- | --- |

This is option 3 for random uncertainty in the flow chart, labeled “no SR feasible”.

Lastly, (12) requires an estimate of random uncertainties in every measurand. We use either

|  | or , | (13) |
| --- | --- | --- |

depending on whether we have multiple trials or a single trial for a measurand.

Advice

## 1. 	Abandon “human error” as a source of uncertainty

The terms uncertainty and error do not imply mistake or blunder. Generally, we find a claim of  “human error” unacceptable as a source of uncertainty. We expect you to execute an experiment correctly and to identify and eliminate outliers before reporting a final resultant.

## 2. 	Perform ”multiple”, not “repeated”, trials.

It is a common misconception that random uncertainty can be reduced by repeating the measurement over a short time span, thereby “eliminating” or “controlling” random variations in the measurand.

Such a procedure produces an optimistically false report.

The goal of uncertainty analysis is to estimate the 95% confidence interval—that 95% of samples would give an interval that includes the true value of the resultant. In other words, the confidence interval is your range of plausible values.

If you repeat measurements over a short time span, you decrease the likelihood that your interval includes the true value—producing a false report.

## 3. 	Assess the contribution of uncertainty sources

Once the sensitivity coefficients, systematic uncertainties in the measurands, and the random uncertainty in the resultant are computed, it is often useful to determine the separate terms given by,

|  | . | (14) |
| --- | --- | --- |

If reducing the total uncertainty is necessary, computing the individual terms in (14) can guide your efforts. The largest of these terms indicates the source that contributes most to the total uncertainty in the resultant.

## 4. 	Choosing the  units of the resultant you report

ASME requires authors to change units of a resultant such that the value reported is between 0.1 and 1000.  For example,

R = 0.002219 m 	becomes 	R = 2.219 mm

R = 19700 sec 	becomes 	R = 5.47 hr

R = 0.0104 lb-ft 	becomes 	R = 2.00 oz-in

However, do not violate convention when applying this rule.  For example, the elastic modulus E is usually reported in units of psi; E = 30106 psi would never be reported as E = 15 kton/in2.

You try:

- R = 0.00000542 m

- R = 5,420 grams

- R = 43,560 ft2

## 5. 	Significant digits rule for the resultant you report

Round the uncertainty to one significant digit unless the leading digit of the uncertainty is a 1 or 2, then round to two significant digits. Report the resultant to the same decimal position. Keep all digits during intermediate calculations; this rule is only for the final result. For example,

126.934 ± 4.229 	becomes 	127 ± 4

9.8237 ± 0.01385 	becomes	9.824 ± 0.014

126.934 ± 0.229 	becomes	126.93 ± 0.23

327.934 ± 41.226 	becomes 	330 ± 40

40 ± 102 	becomes 	40 ± 100 (experiment refinement needed)

You try:

- 572.427 ± 2.541

- 1.0475 ± .0053

- 1,072.31 ± 50.12

- 3.50 ± 0.0000019

This page is intentionally blank.

Practice problem 1

Assume air is an ideal gas. Multiple measurements are made of room air temperature & pressure. Sensor specifications are given. Determine the air density and its uncertainty.

| Data |  |  | Sensor specifications |
| --- | --- | --- | --- |
| T (°C) | P (kPa) |  | thermocouple accuracy: 0.2°C |
| 20.9 | 101.1 |  | readout accuracy: 0.05°C |
| 21.0 | 101.0 |  | readout least significant digit: 0.1°C |
| 21.5 | 101.1 |  | pressure gage accuracy: 0.1 kPa = 100 Pa |
| 20.2 | 101.0 |  | smallest division: 0.1 kPa = 100 Pa |
| 20.7 | 101.1 |  |  |

The data reduction equation (DRE) is:

What case do we have?

The model of uncertainty is:

| ## Resultant | (partial calculations) T (°C); P (kPa); T (K); P (Pa);  (kg/m3); R (J/kgK). 20.9; 101.1; 294.05; 101,100; 1.1980; 287. 21.0; 101.0; 294.15; 101,000; 1.1964. 21.5; 101.1; 294.65; 101,100; 1.1955. 20.2; 101.0; 293.35; 101,000; 1.1996. 20.7; 101.1; 293.85; 101,100; 1.1988. 294.01; 101060; 1.1977; mean. 0.47; 55; 1.692910; std. dev.. |
| --- | --- |
| ## Sensitivity coefficients | ## Answers: ## 4.073610 kg/m3 /K 11.85110 kg/m3 /Pa |
| ## Systematic uncertainty in the measurands | ## Answers: ## 212.1310 K ## 111.80 Pa |

## Random uncertainty in the resultant 	.

|  | ← number of trials ← degrees of freedom ← from Table 6.6 (or use the MATLAB tinv function) |
| --- | --- |

Answer:

2.10210    kg/m3

## Total uncertainty in the resultant.

Substituting  all the individual terms we’ve computed in the uncertainty model:

Answer2.630810  kg/m3.

## Experimental result



Practice problem 2

Assume air is an ideal gas. Multiple measurements are made of room air temperature. A single measurement of air pressure is made but a standard deviation in pressure has been determined from another experiment. Sensor specifications are given. Determine the air density and its uncertainty.

| Data |  |  | Sensor specifications |
| --- | --- | --- | --- |
| Temp. (°C) | Press. (kPa) |  | thermocouple accuracy: 0.2°C |
| 20.9 | 101.1 |  | readout accuracy: 0.05°C |
| 21.0 |  |  | readout least significant digit: 0.1°C |
| 21.5 |  |  | pressure gage accuracy: 0.1 kPa |
| 20.2 |  |  | smallest division: 0.1 kPa |
| 20.7 |  |  |  |
| 20.9 |  |  | Auxiliary pressure-experiment result: 25 Pa |
| 20.8 |  |  | assume N large enough so that t = 2. |

The data reduction equation (DRE) is:

What case do we have?

The model of uncertainty is:

| ## Resultant Answer: 1.1982 kg/m3 | (partial calculations) Temp (°C); Temp (K). 20.9; 294.05. 21.0; 294.15. 21.5; 294.65. 20.2; 293.35. 20.7; 293.85. 20.9; 294.05. 20.8; 293.95. 294.007. 386.6810. |
| --- | --- |
| ## Sensitivity coefficients ## (same expressions as practice problem 1, some mean values are different) | ## Answers: ## 4.075210 kg/m3 /K ## 11.851110 kg/m3 /Pa |
| ## Systematic uncertainty in the measurands (same as practice problem 1) | ## Answers: ## 212.1310 K ## 111.80 Pa |

## Random uncertainty in pressure.

|  |  | Ans.: Pa. |
| --- | --- | --- |

Random uncertainty in temperature

|  |  | Ans.: 357.6210 . |
| --- | --- | --- |

Total uncertainty in the resultant.

Answer2.231210 kg/m3.

## Experimental result



# Workshop 3

Pre-Lab: Flow Measurement Example

# Instruction

Read this handout. Answer questions 1–4. Using a software package is optional.

Figure 1. Schematic of a venturi flow measurement system

Experimental context.

A venturi flow meter is installed in a water piping system.  A pressure transducer measures the pressure difference p between the locations  labeled P1 and P2 . The sensor produces a DC voltage signal proportional to p. The p measurement is used to compute volumetric flow rate, V.

The goal is to determine the volumetric flow rate and its uncertainty,

V±wV.

# Data reduction equation

The data reduction equation is given by

| V=Kd2Δpρ , | (1) |
| --- | --- |

where K is a flow coefficient and   is the water density. Assume that K = 5.054 (no units) and  = 998 kg/m3 are known perfectly. Diameter d has been measured with d  = 25.73 mm ± 0.01 mm.

The two measurands are:  ___________ and ___________ .

The constant parameters are:  ___________ and ___________ .

| # Sensor specifications - The pressure transducer is an Omega #PX409-001. Determine the following characteristics and identify them on the figure, including axis labels & units. - Min & max input values - Min & max output values (with 10 V excitation ) - Sketch the expected calibration curve - Expected sensitivity - Expected calibration equation # Steady-state measurements With the flow at steady state, multiple p measurements are taken over a sufficiently long time span (avoiding the problem of “repeated measurements”). Readings are shown in Table 1. | Expected calibration curve |
| --- | --- |
| - Use the expected calibration equation to determine the measurand (pressure) for each of the readings (1 psi = 6,895 Pa). Complete Table 2. - For each pressure reading, use the DRE to compute a flow rate. Show one example of units analysis. Complete Table 3. - Determine the mean steady-state flow rate. |  |

| Table 1. Pressure transducer readings at steady flow. |  | Table 2. Differential pressure data at steady flow. |  | Table 3. Flow rate results. |
| --- | --- | --- | --- | --- |
| Readings (mV) |  | p (Pa) |  | Flow rate (× m3/s) |
| 1.57 |  | 108.252 |  | 1.1020 |
| 1.48 |  |  |  |  |
| 1.45 |  |  |  |  |
| 1.55 |  |  |  |  |
| 1.63 |  |  |  |  |
| 1.70 |  |  |  |  |
| 1.71 |  |  |  |  |
| 1.46 |  |  |  |  |
| 1.49 |  |  |  |  |
| 1.50 |  |  |  |  |
|  |  |  |  | mean: |
| In a report, a table of raw data like this is in an appendix. Voltage readings do not convey much information to the general reader. |  | In a report, a table of measurand data like this appears in the data section. |  | In a report, a table of results like this appears in the results section. |